# IMU Assignment – Process So Far and Next Steps

## 1. Goal of the IMU Assignment

The goal of this assignment is to build a streaming IMU data pipeline using a Raspberry Pi with a Sense HAT, combined with real-time processing on a laptop. The system follows a Data Flow Architecture (DFA) approach and is designed to be operated step-by-step from a Jupyter Notebook.

Key requirements: - Use the Sense HAT IMU (accelerometer + gyroscope) - Publish data at a fixed sampling interval (20 ms / 50 Hz) - Timestamp every measurement - Stream data using MQTT - Perform processing and estimation on the laptop

## 2. Current System Architecture

### Hardware

- Raspberry Pi with Sense HAT attached

- Laptop running PyCharm + Jupyter Notebook

### Software Components

On the Raspberry Pi: - imu_streamer.py - Reads accelerometer and gyroscope data from the Sense HAT - Publishes IMU measurements to an MQTT broker (Mosquitto) - Sampling rate: 20 ms (50 Hz) - Each message is timestamped (epoch ms + ISO UTC) - Optionally logs data to CSV

On the Laptop: - Jupyter Notebook (run via PyCharm) - Subscribes to the MQTT topic - Buffers incoming IMU messages - Allows cell-by-cell execution for inspection and processing

Communication: - MQTT over local network - Topic used: imu/raw

## 3. What Has Been Achieved So Far

### 3.1 Sense HAT and IMU Verification

- I2C interface enabled and verified

- Sense HAT IMU detected on the I2C bus

- Direct Python test confirmed working IMU:

  - Gyroscope returns valid values

  - Accelerometer initializes correctly after warm-up

### 3.2 Robust IMU Streaming on the Pi

- Implemented retry-based IMU initialization

- Added read retries to prevent crashes

- Implemented warm-up phase before publishing

- Ensured stable publishing at 50 Hz (20 ms)

### 3.3 MQTT Streaming Confirmed

- Mosquitto broker running on the Raspberry Pi

- IMU messages successfully published to imu/raw

- Verified using mosquitto_sub

### 3.4 Jupyter Notebook Integration

- MQTT subscriber implemented inside notebook

- Data arrives asynchronously while notebook cells remain interactive

- Messages are stored in a rolling buffer

- Live inspection of latest IMU samples works

This confirms a fully working streaming data pipeline.

## 4. How the Notebook Is Intended to Be Used

The notebook is deliberately structured so that it can be executed cell by cell:

- Setup cell – imports, configuration, buffers

- Start subscriber cell – starts MQTT client once

- Inspection cell – can be run repeatedly to view live data

- Stop cell – cleanly shuts down MQTT client

- Analysis cells – filtering, estimation, plots

This ensures reproducibility and prevents background processes from interfering with results.

## 5. Data Format (Current)

Each MQTT message on imu/raw contains: - timestamp_ms – epoch timestamp (milliseconds) - timestamp_iso_utc – human-readable UTC timestamp - dt_s – sampling interval (0.02 s) - accel_g.{x,y,z} – accelerometer in g - gyro_dps.{x,y,z} – gyroscope in degrees per second

This format is suitable for direct use in filtering and estimation algorithms.

## 6. Next Steps (Planned Work)

### Step 1 – Orientation Estimation

- Integrate gyroscope z-axis (gyro_dps.z) to estimate yaw angle

- Handle integration drift over time

### Step 2 – Noise Reduction

- Apply a linear Kalman filter to:

  - Gyro rate, or

  - Integrated yaw angle

- Improve stability and reduce measurement noise

### Step 3 – Velocity / Drift Correction

One of the following will be implemented: - Joystick button on Sense HAT to trigger a reset or correction - Stationary detection using accelerometer magnitude

This step ensures long-term stability and satisfies the velocity correction requirement of the assignment.

### Step 4 – Visualization and Analysis

- Plot raw vs filtered signals

- Demonstrate effect of filtering

- Discuss trade-offs (noise vs responsiveness)

## 7. End Result

At completion, the system will demonstrate: - Real-time IMU data streaming - Structured DFA pipeline (sensor → stream → processing) - Online state estimation - Clean separation between data acquisition (Pi) and processing (laptop)

This directly aligns with the DFA and IMU assignment objectives.